Case study from Case study- forecasting-2

Answer to the question no 1:

## a) Given Data

| Year | Demand |
| --- | --- |
| 2017 | 2.00 |
| 2018 | 2.50 |
| 2019 | 2.18 |
| 2020 | 3.20 |
| 2021 | 3.50 |
| 2022 | 3.30 |
| 2023 | 3.45 |

## (i) Three-period Moving Average

Use last 3 years: 2021, 2022, 2023

F2024=3.50+3.30+3.453
=10.253=3.42

Forecast (MA) = 3.42 million

## (ii) Weighted Moving Average

Weights:

- 2023 → 0.5

- 2022 → 0.3

- 2021 → 0.2

F2024=(3.45×0.5)+(3.30×0.3)+(3.50×0.2)
=1.725+0.99+0.70=3.415

Forecast (WMA) = 3.42 million

# (b) Exponential Smoothing

## Given:

- α = 0.4 and 0.5

- Forecast of 2018 = Naive = 2017 actual = 2.00

## Case 1: α = 0.4

Using:

Ft=Ft-1+α(At-1-Ft-1)

| Year | Actual | Forecast |
| --- | --- | --- |
| 2018 | 2.50 | 2.00 |
| 2019 | 2.18 | 2.20 |
| 2020 | 3.20 | 2.19 |
| 2021 | 3.50 | 2.59 |
| 2022 | 3.30 | 2.95 |
| 2023 | 3.45 | 3.09 |
| 2024 |  | 3.23 |

Forecast (α=0.4) = 3.23

Case 2: α = 0.5

| Year | Actual | Forecast |
| --- | --- | --- |
| 2018 | 2.50 | 2.00 |
| 2019 | 2.18 | 2.25 |
| 2020 | 3.20 | 2.215 |
| 2021 | 3.50 | 2.707 |
| 2022 | 3.30 | 3.103 |
| 2023 | 3.45 | 3.201 |
| 2024 |  | 3.33 |

Forecast (α=0.5) = 3.33

Higher α (0.5) responds faster to recent data and data shows upward trend so Best α = 0.5

Answer to the question no 2:

MAD, MSE, MAPE Calculation

Given:

| Year | Actual | Forecast |
| --- | --- | --- |
| 2017 | 2.00 | 2.10 |
| 2018 | 2.50 | 2.30 |
| 2019 | 2.18 | 2.35 |
| 2020 | 3.20 | 2.25 |
| 2021 | 3.50 | 2.80 |
| 2022 | 3.30 | 3.25 |
| 2023 | 3.45 | 3.20 |

| Year | A | F | E= A−F | ¦E¦=∣A−F∣ | E²=(A − F)² | APE=(¦A − F¦ / A)) × 100 |
| --- | --- | --- | --- | --- | --- | --- |
| 2017 | 2.00 | 2.10 | -0.10 | 0.10 | 0.01 | 5.00 |
| 2018 | 2.50 | 2.30 | 0.20 | 0.20 | 0.04 | 8.00 |
| 2019 | 2.18 | 2.35 | -0.17 | 0.17 | 0.0289 | 7.80 |
| 2020 | 3.20 | 2.25 | 0.95 | 0.95 | 0.9025 | 29.69 |
| 2021 | 3.50 | 2.80 | 0.70 | 0.70 | 0.49 | 20.00 |
| 2022 | 3.30 | 3.25 | 0.05 | 0.05 | 0.0025 | 1.52 |
| 2023 | 3.45 | 3.20 | 0.25 | 0.25 | 0.0625 | 7.25 |

## Totals:

∑∣E∣=2.42
∑E2=1.5364
∑APE=79.26

## MAD:

MAD=2.427=0.346

MAD = 0.35

## MSE:

MSE=1.53647=0.219

MSE = 0.22

## MAPE:

MAPE=79.267=11.32%

MAPE = 11.32%

Case study from Case study- forecasting

## Given data

Year Production

2000-01 16.6

2001-02 19.3

2002-03 20.8

2003-04 23.8

2004-05 26.5

2005-06 29.0

2006-07 31.1

2007-08 33.4

2008-09 36.4

2009-10 38.9

2010-11 40.7

For fair comparison, use the common forecast years:
2003-04 to 2010-11.

## Method used for comparison

To compare forecasting accuracy, the standard error measures are used:

MAD=∑∣A-F∣n
MSE=∑(A-F)2n-1
MAPE=∑∣A-F∣A100n

## 1. Three-year moving average accuracy

From the forecast table, the moving-average forecasts for 2003–04 to 2010–11 are 18.9, 21.3, 23.7, 26.43, 28.87, 31.17, 33.63, and 36.23.

| Year | Actual (A) | Forecast (F) | A-F | ¦A-F¦ | (A-F)^2 | APE % |
| --- | --- | --- | --- | --- | --- | --- |
| 2003–04 | 23.8 | 18.9 | 4.90 | 4.90 | 24.01 | 20.59 |
| 2004–05 | 26.5 | 21.3 | 5.20 | 5.20 | 27.04 | 19.62 |
| 2005–06 | 29.0 | 23.7 | 5.30 | 5.30 | 28.09 | 18.28 |
| 2006–07 | 31.1 | 26.43 | 4.67 | 4.67 | 21.81 | 15.02 |
| 2007–08 | 33.4 | 28.87 | 4.53 | 4.53 | 20.52 | 13.56 |
| 2008–09 | 36.4 | 31.17 | 5.23 | 5.23 | 27.35 | 14.37 |
| 2009–10 | 38.9 | 33.63 | 5.27 | 5.27 | 27.77 | 13.55 |
| 2010–11 | 40.7 | 36.23 | 4.47 | 4.47 | 19.98 | 10.98 |

∑(A-F)2=196.57
∑APE=125.96

So,

MAD=39.578=4.95
MSE=196.577=28.08
MAPE=125.968=15.75%

### Result for moving average

MAD = 4.95
MSE = 28.08
MAPE = 15.75%

## 2. Exponential smoothing accuracy

From the exponential smoothing table, the forecasts for the common years 2003–04 to 2010–11 are 17.26, 17.92, 18.78, 19.80, 20.93, 22.17, 23.60, and 25.13.

| Year | Actual (A) | Forecast (F) | A-F | ¦A-F¦ | (A-F)^2 | APE % |
| --- | --- | --- | --- | --- | --- | --- |
| 2003–04 | 23.8 | 17.26 | 6.54 | 6.54 | 42.77 | 27.48 |
| 2004–05 | 26.5 | 17.92 | 8.58 | 8.58 | 73.62 | 32.38 |
| 2005–06 | 29.0 | 18.78 | 10.22 | 10.22 | 104.45 | 35.24 |
| 2006–07 | 31.1 | 19.80 | 11.30 | 11.30 | 127.69 | 36.33 |
| 2007–08 | 33.4 | 20.93 | 12.47 | 12.47 | 155.50 | 37.34 |
| 2008–09 | 36.4 | 22.17 | 14.23 | 14.23 | 202.49 | 39.09 |
| 2009–10 | 38.9 | 23.60 | 15.30 | 15.30 | 234.09 | 39.33 |
| 2010–11 | 40.7 | 25.13 | 15.57 | 15.57 | 242.42 | 38.25 |

∑(A-F)2=1182.53
∑APE=285.44

So,

MAD=94.218=11.78
MSE=1182.537=169.00
MAPE=285.448=35.68%

### Result for exponential smoothing

MAD = 11.78
MSE = 169.00
MAPE = 35.68%

## 3. Least square method accuracy

From the least square table, the forecasts for 2003–04 to 2010–11 are 23.88, 26.33, 28.78, 31.23, 33.69, 36.14, 38.59, and 41.04.

| Year | Actual (A) | Forecast (F) | A-F | ¦A-F¦ | (A-F)^2 | APE % |
| --- | --- | --- | --- | --- | --- | --- |
| 2003–04 | 23.8 | 23.88 | -0.08 | 0.08 | 0.0064 | 0.34 |
| 2004–05 | 26.5 | 26.33 | 0.17 | 0.17 | 0.0289 | 0.64 |
| 2005–06 | 29.0 | 28.78 | 0.22 | 0.22 | 0.0484 | 0.76 |
| 2006–07 | 31.1 | 31.23 | -0.13 | 0.13 | 0.0169 | 0.42 |
| 2007–08 | 33.4 | 33.69 | -0.29 | 0.29 | 0.0841 | 0.87 |
| 2008–09 | 36.4 | 36.14 | 0.26 | 0.26 | 0.0676 | 0.71 |
| 2009–10 | 38.9 | 38.59 | 0.31 | 0.31 | 0.0961 | 0.80 |
| 2010–11 | 40.7 | 41.04 | -0.34 | 0.34 | 0.1156 | 0.84 |

∑(A-F)2=0.4639
∑APE=5.37
So,

MAD=1.808=0.225
MSE=0.46397=0.0663
MAPE=5.378=0.67%

### Result for least square method

MAD = 0.225
MSE = 0.066
MAPE = 0.67%

## 4. Comparison of the three techniques

| Technique | MAD | MSE | MAPE |
| --- | --- | --- | --- |
| Three-year Moving Average | 4.95 | 28.08 | 15.75% |
| Exponential Smoothing | 11.78 | 169.00 | 35.68% |
| Least Square Method | 0.225 | 0.066 | 0.67% |

The least square method gives the lowest MAD, lowest MSE, and lowest MAPE among the three methods. This means it has the smallest average error, the smallest squared error, and the smallest percentage error. Therefore, it is clearly the most accurate forecasting method for this case. So the best method is Least Square Method.

Case study from Case-1-Productivity

## Given Data

The case provides the following average productivity per crew:

| Crew Size | Average Productivity per Crew |
| --- | --- |
| 2 workers | 4,234 square feet per day |
| 3 workers | 5,352 square feet per day |
| 4 workers | 7,860 square feet per day |

ANSWER TO THE QUESTION NO:1

### Productivity per worker

For 2-worker crew:

Productivity per worker=42342=2117

For 3-worker crew:

Productivity per worker=53523=1784

For 4-worker crew:

Productivity per worker=78604=1965

Highest productivity: 2-worker crew = 2,117 sq ft per worker per day

Lowest productivity: 3-worker crew = 1,784 sq ft per worker per day

### Possible explanations:

- A 2-worker crew may coordinate better, move faster, and waste less time.

- A 3-worker crew may create imbalance in task sharing, causing waiting or interference.

- A 4-worker crew has the highest total daily output, but not the highest per-worker productivity, possibly because extra workers reduce individual efficiency.

- Large jobs may require more setup, communication, and supervision time, which lowers productivity per worker.

- Differences in garden layout, difficulty of work, and worker skill mix can also affect the result.

ANSWER TO THE QUESTION NO:2

Although the 2-worker crew has higher productivity per worker, the owner sent a 4-worker crew because the customer needed the garden fixed urgently before the weekend party. The decision was based on speed of completion, not only on worker efficiency. The case says the owner believes large jobs should be assigned to four workers so the job can be completed in a reasonable amount of time.

### Explanation

- A 2-worker crew is more efficient per worker, but total output per day is only:

4234 sq ft/day

- A 4-worker crew can complete:

7860 sq ft/day

So the 4-worker crew can finish much more work in one day.

### Rationale:

The owner likely chose the 4-worker crew because:

- The customer situation was urgent.

- Faster completion was more important than labor efficiency.

- Customer satisfaction and service reliability were critical.

- A late completion could damage the company’s reputation.

So, even if the 2-worker crew is more productive per worker, the 4-worker crew is better when completion time matters most.

ANSWER TO THE QUESTION NO:3

A major qualitative issue is quality of work. The productivity ratios only show how many square feet were completed per day, but they do not show whether the work was done carefully and correctly.

### Explanation:

Two crews may show similar productivity numbers, but one crew may:

- plant more neatly,

- maintain better design quality,

- make fewer mistakes,

- satisfy the customer more.

These important factors are not included in the productivity ratio.

### Other possible qualitative issues:

- Worker skill and experience

- Motivation

- Customer satisfaction

- Weather conditions

- Difficulty of the garden design

- Team cooperation

Thus, productivity ratios are useful, but they do not capture the full performance of a crew.

Case study from Case-2-Capacity_Planning

ANSWER TO THE QUESTION NO:1

### Step 1: Processing time

Mango:

3×140000=420000 min

Orange:

4×120000=480000 min

Papaya:

3×80000=240000 min

Total:

420000+480000+240000=1140000 min

Convert to hours:

1140000÷60=19000 hr

### Step 2: Available time per machine

### 8×240=1920 hr
Step 3: Machines required

190001920=9.895≈10 machines

### Step 4: Extra machines

10-5=5

Extra machines required = 5

ANSWER TO THE QUESTION NO:2

### Step 1: With 80% efficiency

1920×0.8=1536 hr
190001536=12.36≈13 machines

### Step 2: With 90% efficiency

1920×0.9=1728 hr
190001728=10.99≈11 machines

### Step 3: Extra machines

11-5=6

So, by increasing 10% efficiency company require 11-5 = 6 extra machines

ANSWER TO THE QUESTION NO:3

### Step 1: Demand(Q)

140000+120000+80000=340000 per year
340000÷12=28333 per month

### Step 2: Costs

Fixed cost(FC):

5000+20000=25000

Variable cost(v):
2 per unit

Purchase cost(R):

5 per unit
Step 3: Break-even point

Q (BEP) = FC/R-V =250005-2=250003=8333
Step 4: Cost comparison

In-house:

FC + vQ=25000+2×28333=81666
Supplier:

RQ=5×28333=141665
Since

81666<141665
So, in-house production cost is lower. Hence, company should go for in-house production